Developer Guide

Custom Development Board Adaptation Guide - Pin configuration for new hardware, driver development, compatibility testing.

Agent Integration Tutorial - Third-party agent access, function expansion, interaction optimization.

Firmware Upgrade Development Guide - Custom OTA process, firmware signing, version management.

Multilingual Extension Tutorial - Adding new language packs, voice model adaptation, translation function configuration.

System Configuration

Testing Environment

WebSocket Connection Address: wss://api.apsets.com/agentTestWS/v2/chat

Production Environment

WebSocket Connection Address: wss://api.apsets.com/agentWS/v2/chat

Related Open Source Projects

Server-side Projects

future-explore/iot-ws-server - Golang version of the WebSocket streaming dialogue service.

future-explore/iot-management-backend - Device management backend service.

Client Projects

future-explore/iot-ios-client - iOS device control APP.

future-explore/iot-linux-client - Linux device client.

About the Project

This project is an open-source IoT intelligent dialogue solution by Future Explore, released under the MIT license. It allows free use, modification, and secondary development for both personal and commercial purposes. We are committed to lowering the development threshold for IoT intelligent interaction by providing standardized protocols and a modular architecture, helping developers quickly achieve intelligent upgrades for their devices.

Hardware adaptation list

| Hardware model | Core chip | Key hardware parameters | Pin Mapping (Core Function) | List of Supported functions | Adaptation Precautions | Reference materials link |
| --- | --- | --- | --- | --- | --- | --- |
| Lichuang · Practical ESP32-S3 Development Board | ESP32-S3-WROOM-1 | 4MB Flash + 2MB PSRAM, 1.14-inch LCD (240*135), mono microphone + speaker interface | Audio Input: GPIO34 (Microphone) - Audio Output: GPIO21 (DAC) - LCD driver: SPI2 (GPIO12/13/14/15) - Keys: GPIO0 (Reset), GPIO1 (User key) | ✅ Wi-Fi connection ✅ Offline voice wake-up ✅ Streaming ASR/LLM/TTS ✅ LCD expression display ✅ Volume control ✅ Scheduled task reminder | 1.The speaker needs to be connected to a 3.3V power amplifier module externally. There is no sound when directly connected to the DAC pin. 2. When burning, pull the BOOT pin (GPIO9) low, reset it and then release it | The project page of Lichuang Open Source Platform |
| Espressif ESP32-S3-BOX3 | ESP32-S3-N16R8 | 16MB Flash + 8MB PSRAM, 2.4-inch IPS LCD (320*240), dual microphone array + 2W speaker, integrated PIR human body sensor | Audio input: I2S0 (GPIO42/41/40, dual microphone) - Audio output: I2S1 (GPIO17/18, speaker) - LCD driver: SPI3 (GPIO5/6/7/8) - PIR sensor: GPIO21 | ✅ Wi-Fi/Bluetooth 5.0 ✅ Voiceprint recognition (3D Speaker) ✅ Multimodal interaction (voice + display) ✅ Human body induction wake-up ✅ OTA online upgrade ✅ Intelligent home control ✅ Power monitoring | 1. The factory defaults to burning the official firmware of Espressif. You need to erase the Flash first before burning the firmware of this project. 2. The PIR sensor is turned off by default. You need to enable the GPIO21 interrupt function in the code | Espressif official documentation |

If you have any ideas or questions, feel free to add our WeChat or join the QQ discussion group.

Technical Support

- Official QQ Group: 1056381308

- Documentation Center:

- Business Cooperation: nextbigseek@nextbigseek.com